- Stretchable ionic cable and pressure sensor based on rapid self-recoverable and tough conductive hydrogel

Abstract Title (Time New Roman, bold, 14 point)

- Yong Mei Chen1,2, Dian Dian Dong1, Mei Xiang Wang2

- Presenting Author, Authors (Time New Roman, 12 point)

（1Key Laboratory of Leather Cleaner Production, China National Light Industry, National Demonstration Center for Experimental Light Chemistry Engineering Education, College of Bioresources Chemical and Materials Engineering, Shaanxi University of Science & Technology, Xian 710021, China; 2Xian Jiaotong University, Xian 710049, China.）

Affiliations (Time New Roman, Italic, 10.5 point)

- *E-mail: 913898005@qq.com

E-mail address (Time New Roman, 10.5 point)

Hydrogels as soft and wet materials have attracted much attention in sensing and flexible electronics. However, traditional hydrogels are fragile or have unsatisfactory recovery capability, which largely limit their applications. A novel hydrogen bond based sulfuric acid−poly(acrylic acid) (PAA)/poly(vinyl alcohol) physical hydrogel is developed for addressing the above drawbacks. The hydrogel obtains comprehensive mechanical properties, including extremely rapid self-recovery, high fracture strength (3.1 MPa), and high toughness (18.7 MJ m−3). We demonstrate this hydrogel as a stretchable ionic cable and pressure sensor to exhibit stable operation after repeated loadings. This work provides a new concept to synthesize physical hydrogels, which will hopefully expand applications of hydrogel in stretchable electronics.

(Times New Roman 12 point) Each abstract for the preprints should be prepared in .doc or .docx format (or edited by Microsoft Word) within 1 pages of A4.

Keywords: Gel, Self-recovery, Conductivity, Pressure sensor, Toughness

- References:

- [1]. Wei Z.; Yang JH.; Liu ZQ.; Xu F.; Zhou JX.; Zrínyi M.; Osada Y.; Chen YM* ,  Adv. Funct. Mater. 2015, 25,1352

This research was supported by the National Natural Science Foundation of China (Grant No. XXXX).